TT-DA91T-SL

Laser High Speed Dome

……………………………………………………………………………………………………

Features

Laser night vision:

- * Adopt Laser light+ Secondary Concentrator Bilens, night vision distance up to 200 meters.

- * According to the zoom, open the laser IR light in sections, night vision effect evenly.

- * With Laser light’s strong ability to penetrate the fog, it solves the problems for the night fog in the outside.

PTZ function

- Alarm(Optional)

- Adaptive PELCO-P/D protocol, Auto detected Baud Rate

- Auto running memory against power outage

- RTC function, call any functions at any time

- 8 privacy masks (need the support of the camera)

- Support 128 preset positions, and support lens preset

- English OSD menu, Short-cut key operation to PTZ functions

- Pan/Tilt speed: 0.08°～240°/S, preset accuracy: <±0.1°

- Soft Address: can modify the 485 address remotely, convenient installation and maintenance

- Compass function,proportional zoom and auto flip function

- Built in double devices for lightning proof and surge proof

- Support multi-protocols: SONY、LG、CNB、HITACHI、SAMSUNG etc.

- Support 4 groups cruise scan,4 group patterns, one scan, and 360°continuous scan and 360°intermittent scan

Standard camera module

* DSP ：SONY EFFIO-E；

* 1/3" SONY CCD；

* 32X, 700TVL, 36X 650TVL, 36X 800TVL

*Focal Length：5.4～97.2mm；

* IR-Cut Switch；

Specifications

| Power Supply | Output：DC12V±5%，5A Input：AC100～240V 50/60Hz |
| --- | --- |
| Max. Power | IR off：15W IR on：35W |
| Sync system | Internal Sync |
| Language | English |
| Pan Speed | 0.08°～240°/S |
| Tilt Speed | 0.08°～240°/S |
| Pan Range | 0°～360°(Continuous) |
| Tilt Range | 0°～90°（180° auto flip） |
| Preset | 128 |
| Preset Accuracy | <±0.1° |
| Tour | 4, 27 presets/tour |
| Pattern | 4 |
| Zone | 1 |
| Protocol | RS485 |
| Communication | PELCO-D/P auto Adaptive |
| Baud Rate | 2400bps、4800bps、9600bps、19200bps |
| Address | 1-255 |
| RTC | Set any key at any time |
| IR Light switch | Default::1~5 times, open the near-distance lights; 6~9 times, open the near-distance and middle-distance lights; after 10 times, turn of f the near-distance lights, open the middle-distance and far-distance lights. (Light open times can be settable, near-distance light close and far-distance light open happen at the same time. |
| Work Temperature | Indoor：0℃～+50℃； Outdoor ：-20℃～+50℃(with heater) |
| Work Humidity | <90% |

Size（mm）